# Pipe Pile Optimization: Excel Output Guide

# Overview

The file pipe_optimization_V2_VERIFIED.xlsx contains the complete, verified cutting plan for assembling 262 optimal 100-foot pipe piles from your inventory. Each pipe is assigned a unique index to prove no duplicate usage.

# File Contents

The Excel workbook contains three sheets:

## Sheet 1: Summary

High-level results and metrics including method, status, total piles, pipes used/unused, waste statistics, and comparison to greedy baseline.

## Sheet 2: Pile Details

Complete construction guide with one row per pipe segment:

| Column | Description |
| --- | --- |
| Pile Number | Unique ID for each pile (1-262) |
| Segment | Segment position within the pile (1, 2, or 3) |
| Pipe Index | Unique identifier (0-757) for the specific pipe to use |
| Actual Length (ft) | Exact length of this pipe from inventory |
| Rounded Length (ft) | Length rounded to 0.1' (used in optimization) |
| Total Length | Combined length of all segments (first row only) |
| Waste (ft) | Excess over 100' (first row only) |
| Welds | Number of welds needed (first row only) |

## Sheet 3: Unused Pipes

Lists the 8 pipes that are not used in the solution, with their Pipe Index and Length.

# Verifying No Duplicate Pipe Usage

To confirm each pipe is used at most once:

Open pipe_optimization_V2_VERIFIED.xlsx

Go to Pile Details sheet

Select the Pipe Index column

Sort A-Z (ascending)

Scroll through - each number 0-757 should appear at most once

Cross-reference with Unused Pipes sheet for the 8 missing indices

## Expected Result

750 unique pipe indices in Pile Details

8 pipe indices in Unused Pipes

Total: 758 (matches inventory)

No index appears twice

# How to Use the Cutting Plan

## Step 1: Label Your Pipe Inventory

Before starting, label each physical pipe with its index (0-757):

Open your original inventory file (pipe_lengths_clean.csv)

Row 1 = Pipe Index 0, Row 2 = Pipe Index 1, etc.

Physically tag or mark each pipe with its index number (e.g., "P0", "P1", ... "P757")

## Step 2: Read the Pile Details Sheet

Each pile spans 2-3 rows in the spreadsheet. For example, Pile 1 might show:
• Segment 1: Pipe #757 (51.6')
• Segment 2: Pipe #617 (48.5')
• Total: 100.1', Waste: 0.1', 1 weld

This means: Get pipe #757 and pipe #617, weld them together.

## Step 3: Assembly Workflow

Option A: Sequential Assembly
Start at Pile 1 and work through to Pile 262, locating pipes by their index numbers.

Option B: Batch by Similar Patterns
Filter the spreadsheet to find repeated patterns (e.g., many piles use 50.3' + 50.2') and assemble in batches for welding efficiency.

Option C: Prioritize by Weld Count
Sort by Welds column. Build all 1-weld piles first (36 piles), then 2-weld piles (226 piles).

## Step 4: Track Progress

Add a "Status" column to track completion. Mark each pile as Pending, In Progress, or Complete.

# Working with the Excel File

## Filtering by Pipe Index

To find which pile uses a specific pipe: Click filter dropdown on Pipe Index column, enter the pipe number, and it shows which pile that pipe belongs to.

## Sorting Options

| Sort By | Purpose |
| --- | --- |
| Pile Number | Sequential assembly order |
| Pipe Index | Verify no duplicates |
| Actual Length | Group similar-length pipes |
| Waste | See most/least efficient piles |
| Welds | Prioritize simpler assemblies |

## Printing for Shop Floor

Set print area to Pile Details sheet

Use Page Layout > Print Titles to repeat header row

Print in landscape for better readability

Include Pipe Index column prominently

# Understanding Unused Pipes

The Unused Pipes sheet lists 8 pipes not included in any pile:

| Pipe Index | Length (ft) |
| --- | --- |
| 44 | 20.6 |
| 78 | 22.0 |
| 192 | 26.8 |
| 236 | 28.1 |
| 409 | 35.0 |
| 433 | 36.1 |
| 471 | 38.1 |
| 537 | 41.4 |

These pipes cannot be combined with remaining available inventory to form valid 100-120' piles. They can be saved for future projects, used for other purposes, or combined with new inventory later.

# Troubleshooting

## "I can't find pipe #X in my inventory"

The Pipe Index corresponds to the row number (0-indexed) in your original pipe_lengths_clean.csv file. Pipe Index 0 = Row 1 in CSV, Pipe Index 100 = Row 101 in CSV.

## "The actual length doesn't match my measurement"

Small variations are expected. The optimizer uses lengths rounded to 0.1'. A pipe listed as 50.3' could physically measure 50.25' to 50.34'.

## "A pipe is damaged/missing"

Find a substitute pipe of similar length (within 0.1'). The pile total will change slightly but should remain valid (100-120'). Document the substitution.

## "I want to verify the math myself"

For any pile: Look up each Pipe Index in the original CSV to get exact lengths, sum the Actual Length values, confirm total is between 100.0' and 120.0', and calculate Waste = Total - 100.0'.

# Quick Reference

| Statistic | Value |
| --- | --- |
| Total Piles | 262 |
| 2-pipe piles (1 weld) | 36 |
| 3-pipe piles (2 welds) | 226 |
| Total welds needed | 488 |
| Pipes consumed | 750 |
| Pipes leftover | 8 |
| Average waste | 0.08' per pile |
| Total waste | 20.2' |

## Weld Calculation

36 piles × 1 weld = 36 welds

226 piles × 2 welds = 452 welds

Total: 488 welds

# Files Reference

| File | Purpose |
| --- | --- |
| pipe_optimization_V2_VERIFIED.xlsx | Main output with pipe indices |
| pipe_lengths_clean.csv | Original inventory (index reference) |
| pipe_optimizer_v2.py | Optimization script |
| METHODS.docx | Technical methodology documentation |
| INSTRUCTION_GUIDE.docx | This file |